SMD Star Badge build Instructions for Reflow Oven, Sept 15, 2017

Safety:

- Wear safety glasses as solder splatters can cause permanent eye damage.

- Cover any open cuts with band-aids and wash hands with soap afterwards to avoid lead exposure.

- Tie long hair back and be careful of hot irons/ovens/etc.

- Clean up afterwards. Thanks!

Guide to Assembly (see photo on back)

Front of Board:

- Using the custom stencil and jig, apply solder paste to the board

- Using tweezers carefully pick up and lightly place parts into their spots on the board.

- Make sure parts are carefully aligned especially the chips.

- Try not to move around the paste too much as it holds the parts in place.

- Follow the VHS reflow oven instructions to bake your board.

- Carefully remove board with tweezers.

- Let cool.

Back of Board:

- Hand-solder the battery clip.

- Hot glue the badge pin.

- Insert battery.

Board Schematic:

Star Board designed by Tom Keddie

Place Parts as they appear in the order below.  Check all your parts first!

| Part | Value/Type | Specific Instructions | Packaging Details |
| --- | --- | --- | --- |
| R1 | 10K ohms |  | red |
| R2 | 4.7k ohms |  | pink |
| R3 | 220 ohms |  | blue |
| C1, C2 | 10 uF |  | black |
| IC 1 | Bipolar timer, SO8 | Text reads toward top of board | small chip |
| IC 2 | Divider/counter, S016 | Text reads toward top of board | big chip |
| LEDs 1 - 8 | 805 | Side with tiny green dot. Faces up. Dotted end at star tip | Strip with green T’s. Extras incl. |
| Bake board in oven and allow to cool before proceeding. |  |  |  |
| Battery holder (back) |  | hand-solder |  |
| Badge clip (back) |  | Hot glue after baking front |  |
| Battery 2032 (back) | 3 volt |  |  |

Useful Links:
https://talk.vanhack.ca/t/vhs-smd-reflow-oven/5727 (oven instructions)
https://www.youtube.com/watch?v=_5lksMvmqQc  (watch smds cooking)